[image: image3.emf][image: image4.png]


     Marble Cutter
       MC1200  MC1400
[image: image5.emf]
[image: image6.emf]
[image: image7.emf]                                               
GB
Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.
TECHNICAL SPECIFICATIONS                                        

	Model
	MC1200

	MC1400

	Rated Voltage
	220-240V ~ 50/60HZ
	220-240V ~ 50/60HZ

	Power
	1200w
	1400W

	Max cutting depth 
	90°: 34mm  
	90°: 34mm

	No load speed
	13000/min 
	13000/min

	Blade size 
	φ110Xφ20mm
	φ125Xφ20mm

	Net weight
	2.8kg 
	2.94kg 


Over view 

[image: image8.emf]
A. On/Off switch 
B. Locking button 
C. Handle 

D. Cutting depth scale 

E. Winged screw for cutting depth setting 

F. Protective guard 
G. Winged screw for cutting angle setting (only for MC1400)
H. Cutting angle scale 

J． Base plate  

General Safety Instructions for Electric Tools

This device is exclusively for private use! It is not fit for commercial use!

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury:

SAVE THESE INSTRUCTIONS

1)Work area

• Keep work area clean and well lit. Cluttered and dark areas invite accidents

• Do not operate machines in explosive atmospheres, such as in the presence of flammable liquids, gases or dust. Machines create sparks which may ignite the dust or fumes.

• Keep children and bystanders away while operating a machine. Distractions can cause you to lose control.
2)Electrical safety

• Machine plugs must match the outlet. Never modify the plug in any way. Do not use any adapter plugs with earthed (grounded) machines. Unmodified plugs and matching outlets will reduce risk of electric shock.

• Avoid body contact with earthed or grounded surfaces such as pipes, radiators, ranges and refrigerators. There is an increased risk of electric shock if your body is earthed or grounded.

• Do not expose machines to rain or wet conditions. Water entering a machine will increase the risk of electric shock.

• Do not abuse the cord. Never use the cord for carrying, pulling or unplugging the machine. Keep cord away from heat, oil, sharp edges or moving parts. Damaged or entangled cords increase the risk of electric shock.

• When operating a machine outdoors, use an extension cord suitable for outdoor use. Use of a cord suitable for outdoor use reduces the risk of electric shock.

3)Personal safety

• Stay alert, Watch what you are doing and use common sense when operating a machine. Do not use a machine while you are tired or under the influence of drugs, alcohol or medication. A moment of inattention while operating machines may result in serious personal injury.
• Use safety equipment. Always wear eye protection. Safety equipment such as dust mark, non-skid  safety shoes, hard hat, or hearing protection used for appropriate conditions will reduce personal injuries.

•Avoid accidental starting. Ensure the switch is in the off position before plugging in. Carrying machines with your finger on the switch or plugging in machines that have the switch on invites accidents.

•Remove any adjusting key or wrench before turning the machine on. A wrench or a key left attached to a rotating part of the machine may result in personal injury.

•Do not overreach. Keep proper footing and balance at all times. This enables better control of the machine in unexpected situations.

• Dress properly. Do not wear loose clothing or jewellery. Keep your hair, clothing and gloves away from moving parts. Loose clothes, jewellery or long hair can be caught in moving parts.

• If devices are provided for the connection of dust extraction and collection facilities, ensure these are connected and properly used. Use of these devices can reduce dust related hazards.

4)Machine use and care

• Do not force the machine. Use the correct machine for your application. The correct machine will do the job better and safer at the rate for which it was designed.

•Do not use the machine if the switch does not turn it on and off. Any machine that can not be controlled with the switch is dangerous and must be repaired.

•Disconnect the plug from the power source before making any adjustments, changing accessories, or storing machines. Such preventive safety measures reduce the risk of starting the machine accidentally.

• Store idle machines out of the reach children and do not allow persons unfamiliar with the machine or these instructions to operate the machine. Machines are dangerous in the hands of untrained users.

• Maintain machines. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the machines operation. If damaged, have the machine repaired before use. Many accidents are caused by poorly maintained machines.

•Keep cutting tools sharp and clean. Properly maintained cutting tools with sharp cutting edges are less likely to bind and are easier to control.

• Use the machine, accessories and tool bits etc., in accordance with these instructions and in the manner intended for the particular type of machine, taking into account the working conditions and the work to be performed. Use of the machine for operations different from intended could result in a hazardous situation.
5) Service

• Have your machine serviced by a qualified repair person using only identical replacement parts. This will ensure that the safety of the machine is maintained.

Safety instructions for marble cutters 

· Always use guard provided with the tool. The guard must be securely attached to the power tool and positioned for maximum safety, so the least amount of wheel is exposed towards the operator. Position yourself and bystanders away from the plane of the rotating wheel. The guard helps to protect operator from broken wheel fragments and accidental contact with wheel.

· Use only flat reinforced or diamond cut-off wheels for your power tool. Just because an accessory can be attached to your power tool, it does not assure safe operation.

· The rated speed of the wheel must be at least equal to the maximum speed marked on the power tool. Wheels running faster than their rated speed can break and fly apart.

· Wheel must be used only for recommended applications. 

· The outside diameter and the thickness of your wheel must be within the capacity rating of your power tool. Incorrectly sized wheel cannot be adequately guarded of controlled.

· The arbour size of wheels and flanges must properly fit the spindle of the power tool. Wheels and flanges with arbour wholes that do not match the mounting hardware of the power tool with run out of balance, vibrate excessively and may cause loss of control.

· Wear personal protective equipment. Depending on application, use face shield, safety goggles or safety glasses.
 OPERATION INSTRUCTIONS                                         
Switch On and Off 

To start the machine, press the On/Off switch A.

To lock on, push and hold the On/Off switch A and press the lock-on button B.
To switch the machine off, release the On/Off switch A or press and then release it.

Changing the tool 

Before any work on the tool itself, pull the power plug.
Inserting

Place the mounting flange L onto the motor spindle K 

Place the diamond cutting disc M onto the motor spindle K and tighten the damping screw Q must thereby hold the clamping flange N tightly with the ring spanner P.

When mounting, make sure that the direction of the arrows on the protective guard F and the diamond cutting disc M both point in the same direction.

Removing

Hold the clamping flange N with the ring spanner P and loosen the clamping screw Q using a socket wrench R.
Warning: left-hand thread

Adjusting the cutting angle

Loosen the winged screw G. Pivot the machine sideways until the desired cutting angle is set on scale H. Tighten winged screw G again. Adjusting the cutting depth

To adjust the cutting depth(only for MC1400):

Loosen the winged screw E and displace the base plate J along its front side, lowering or raising it until reaching the desired measure 

Raise: for smaller cutting depths.

Lower: for greater cutting depths.

Use a ruler, scale or something similar to adjust the required depth. Retighten the winged screw E.

Maintain a uniform feeding speed during the entire cutting procedure. 

If a perfect cut is desired, the diamond-cutting disc must protrude 2mm out of the material.
[image: image1.png]



Maintenance 

1. Before daily start-up inspect tool, power cord and saw blade, looking for chips and cracks.
2. If damage has occurred, repair affected ares before use.
3. Clean and blow off the exterior of tool and around air intake vents to remove any standing water or stone dust on the field coil, carbon brusher and bearings. Failure to do this will shorten the life of your tool, and could void your warranty. 
MC1200 ,MC1400 exploded view 
[image: image2.jpg]A TIEIL

MC110B(Z1E-HH-110B)





                     MC1200, MC1400 Part list 
	No.
	Part list
	No.
	Part list

	1
	Wind guide 
	35
	Inner flange 

	2
	O-ring
	36
	Seal ring

	3
	Screw 
	37
	Bearing cover 

	5
	Stator 
	38
	Bush 

	8
	Insulation washer 
	39
	Bearing 

	9
	Bearing 
	40
	Bearing seat 

	10
	Anti-dust cover
	41
	Big gear 

	11
	Handle cover
	42
	Bearing 

	12
	Switch cover
	43
	Guard 

	14
	Switch 
	44
	Pinion gear 

	15
	Spindle 
	45
	Bearing 

	17
	Housing 
	46
	Seal ring 

	18
	Nameplate 
	47
	Washer

	19
	Tapping screw 
	48
	Key 

	21
	Tapping screw 
	49
	Rotor

	22
	Washer 
	50
	Screw

	23
	O-ring 
	51
	Flange screw

	28
	Tapping screw 
	52
	Out flange 

	29
	Cable clamp 
	53
	Spring pin 

	30
	Cable sleeve 
	54
	base

	31
	Power cable 
	55
	Socket 

	32
	Brush holder
	56
	Circlip 

	33
	Carbon brush 
	57
	Wing screw for depth 

	34
	Brush cover
	
	

















