Hot Gun

HG2000

GB

Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.

TECHNICAL SPECIFICATIONS

| Model | HG2000 |
| --- | --- |
| Rated Voltage | 220-240V ~ 50/60Hz |
| Power | 2000W |
| Temperature: | I:400℃ . II:600℃ |
| Air flow: | I:300 l/min . II:500 l/min |
| Net weight | 0.7kg |

Over view

- Temperature change Switch lever;

- Nozzle

- Front Cover

- Housing

- Rear Cover

# GENERAL SAFETY INSTRUCTIONS FOR ELECTRIC TOOLS

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury.  SAVE THESE INSTRUCTIONS

1. Keep work area clean. Cluttered areas and benches invite accidents.

2. Consider the environment of the working area.

- Do not expose tools to rain.

- Do not use tools in damp areas.

- Keep work area well lit.

- Do not use tools if there is risk of fire or explosion.

3. Protection against electric shock

- Avoid body contact with surfaces grounded (eg pipes, radiators, hobs, refrigerators)

4. Keep children away

- Do not allow spectators to touch the tool or power cable or extension. It is recommended to keep all spectators away from the work area.

5. Store tools in a safe place.

- When not used, it is recommended to store the tools out of reach of                                      children. Allow the tool to cool down (approximately 30 minutes after use)

6. Do not force the tool

- The tool will work better and safer if you do not overheat the working surface more than necessary.

7. Use the appropriate tool

- Do not force tools or accessories. Do not work for long periods, to avoid excessive heating. Do not use tools for unscheduled work. Use the nozzles provided and make limited trials to determine the temperature and working distance.

WARNING!

This is not a hairdryer. Severe burns may result from such use.

8. Dress properly

- Do not wear loose clothing or flammable materials that could present a risk in case of accidental contact with hot parts.

- Pay attention to jewelry chains and bracelets that are conductive.

9. Use personal protective equipment

- Use appropriate gloves (PPE) when handling hot parts.

- Use face protection equipment (PPE) if the work requires.

(PPE: Individual Protection Equipment)

10. Clean nozzles

- Accumulation of waste products from stripping could ignite.

11. Do not damage the power cable

- Never carry the tool by the cable or pull the cable off the socket.

- Keep the cable away from sources of heat, oil and sharp edges.

12. Do not over-reach

- Always keep a stable position and balance.

13. Maintain tools

- Maintain tools for better and safer performance. Periodically check the power cable and, if damaged, have it replaced by a repairer. Periodically check the extension of the power cable and replace if damaged.

- Keep the handles clean and free of oil.

14. Disconnect tools

- When not in use or changing accessories such as nozzles, ensure that the tools is disconnected form the mains.

15. Avoid accidental starting

- Ensure that the switch is turned off before connecting the power supply.

16. Use extension cords for outdoor

- When the tool is used outdoors, use only extension cords designed and intended for outdoor use.

17. Stay alert! Use common sense. Do not operate tool when you are tired.

18. Check damaged parts

- Before re-using the tool, it is recommended to carefully inspect the tool

to determine if it can work properly and perform its function.

- Check moving parts, it is recommended to properly repair or replace any damaged or worn parts.

- Defective switches must be replaced by an authorized service agent. Do not use the tool if the switch is faulty.

Warning!

- The use of any accessories other than those recommended in the instructions, may result in injury.

- Do not insert anything into the outlet tube of air.

19. Repairs must be carried out by qualified personal.

This power tool complies with appropriate safety rules. Repairs should be carried out only by qualified personnel and using original spare parts, otherwise it may result in a significant danger to the user.

SPECIAL SAFETY FOR HOT AIR GUN:

Warning!  1. Disconnect the tool from the power source before making any adjustments or maintenance.

Warning!  2. Do not touch the hot nozzle. Wear protective gloves. .

3. When using an extension cord, ensure that a 10A rated cord is used otherwise damage to the tool may occur. Ensure that the cord is fully unreeled before use. If used outdoors a heavy duty type must be used. Have a registered electrician replace or repair any damaged or worm cords immediately.

4. Do not use tool in a confined space where body movement may be restricted.

5. Do not force the tool. It will do a better job and more safely when used at a rate for which it is designed.

6. The voltage must coincide with the directions of the plate.

7. Disconnect the plug from the mains before carrying out work on the device.

8. The power cord and plug must be in perfect condition. Replace damaged parts immediately.

9. Never run the hot air flow towards people or animals.

10. The heat gun should not be used as a hair dryer.

11. Never use the device in the presence of an explosive atmosphere.

12. Before beginning work, prepare a safe place to place the hot air gun.

13. The careless use of the device may cause a fire. Do not use the device in the presence of an explosive environment.

14. The heat can be transferred to the combustible materials that are out of sight.

15. Do not leave the appliance unattended.

16. The stripper can be used upright for hands free to work. A portion of the folding handle is used as support.

17. Do not touch the hot nozzle. Wear protective gloves.

18. If you work on a conductor of heat, ensure good stability and a suitable distance from the point of work.

19. Make sure in all cases that the unit is completely cooled before storing.

20. The heat gun is not a toy, keep it away from children.

21. Be careful when using the heat gun in places where there are combustible materials.

22. Place the tool on its stand after use and allow to cool before storing.

23. This tool is not intended to be used by persons without experience or knowledge.

24. Keep tool away from children.

Warning! Do not smoke, eat or drink in a work area that could or

maybe contaminated by lead.

Warning!

1. A fire may arise if the appliance is not used with care. Always ensure the workplace is well ventilated.

2. Be careful when using the appliance in places where are combustible materials. Never use the tool in the proximity of highly inflammable gases or materials.

3. The machine must not be damp and must not be operated in damp surroundings.

4. Never use the tool as a hair drier.

5. Do not touch the hot nozzle. Allow the tool to cool down completely before storage.

6. Do not leave the machine unattended.

7. Keep the tool and accessories out of the reach of children.

8. Never direct the air stream at persons or animals.

9. Before each use of the machine, check the cable and plug. If damages are detected, do not use the machine. Have repairs performed only by a qualified technician.

10. Never open the machine yourself.

Double insulation for safer operation, it is not necessary to connect to a socket with earth.

OPERATION INSTRUCTIONS:

1. Removing paint: This heat-generating tool can soften all kinds of organic paints.

2. Bending plastic tubes: Heat all around the part to be bent of the plastic tube until it feels no resistance or little resistance, then the plastic tube can be easily bent.

3. Contracting package: Use heat-contracting plastic film to pack things and heat the plastic film at some distance until it gets contracted and tensed.

4. Contracting the heat-contracting tube: The heat-contracted tube used to assemble the electrical products can be quickly heated and quickly contracted.

5. Loosening the tightened screws, belts and auxiliaries.

6. Remove plastic heat-printing.

HOW TO USE THE HOT AIR GUN

WARNNING: Ensure that the Hot Air Gun is switched OFF (O)

before connecting it to the power supply or when attaching any nozzles to it!

"ON/OFF" SWITCH

The On/Off switch has three settings

Position "O" is "OFF" and the other two positions provide different rates

of air flow. Slide the Switch up to position I or II. The hot air gun will

provide different air flow and temperature as below:

|  | AIRFLOW ANDTEMPERATURE |
| --- | --- |
| Stage I: | 400℃, 300L/min |
| Stage II: | 600℃, 500L/min |

To switch "OFF" the Hot Air Gun, slide the On/Off Switch to the "O" position.

# MAINTENANCE

Warning! Be sure to switch power off and disconnect the plug.

To extend the working life of your tool you must have the tool serviced regularly by an authorized servicing agent.

Warning! Never immerse the machine and do not use detergents, alcohol, gasoline, etc…

# ENVIRONMENTAL PROTECTION

Electrical products should not be disposed of with household waste.

Please recycle where facilities exist.

Check with your local authority or retailer for recycling advice.

HG2000 exploded view

HG2000 part list