Palm Sander

PS350

GB

Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.

TECHNICAL SPECIFICATIONS

| Model | PS350 |
| --- | --- |
| Rated Voltage | 220-240V ~ 50/60Hz |
| Power | 350W |
| No load speed | 12000/min |
| Sanding pad size | 110X100mm |
| Net weight | 1.15kg |

Over view

1. ON/OFF switch                           2. Handle

3. Dust bag                                4.clamp lever

5.clamp spring                             6.Sole

General Safety Instructions for Electric Tools

This device is exclusively for private use! It is not fit for commercial use!

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury:

SAVE THESE INSTRUCTIONS

1)Work area

• Keep work area clean and well lit. Cluttered and dark areas invite accidents

• Do not operate machines in explosive atmospheres, such as in the presence of flammable liquids, gases or dust. Machines create sparks which may ignite the dust or fumes.

• Keep children and bystanders away while operating a machine. Distractions can cause you to lose control.

2)Electrical safety

• Machine plugs must match the outlet. Never modify the plug in any way. Do not use any adapter plugs with earthed (grounded) machines. Unmodified plugs and matching outlets will reduce risk of electric shock.

• Avoid body contact with earthed or grounded surfaces such as pipes, radiators, ranges and refrigerators. There is an increased risk of electric shock if your body is earthed or grounded.

• Do not expose machines to rain or wet conditions. Water entering a machine will increase the risk of electric shock.

• Do not abuse the cord. Never use the cord for carrying, pulling or unplugging the machine. Keep cord away from heat, oil, sharp edges or moving parts. Damaged or entangled cords increase the risk of electric shock.

• When operating a machine outdoors, use an extension cord suitable for outdoor use. Use of a cord suitable for outdoor use reduces the risk of electric shock.

3)Personal safety

• Stay alert, Watch what you are doing and use common sense when operating a machine. Do not use a machine while you are tired or under the influence of drugs, alcohol or medication. A moment of inattention while operating machines may result in serious personal injury.

• Use safety equipment. Always wear eye protection. Safety equipment such as dust mark, non-skid  safety shoes, hard hat, or hearing protection used for appropriate conditions will reduce personal injuries.

•Avoid accidental starting. Ensure the switch is in the off position before plugging in. Carrying machines with your finger on the switch or plugging in machines that have the switch on invites accidents.

•Remove any adjusting key or wrench before turning the machine on. A wrench or a key left attached to a rotating part of the machine may result in personal injury.

•Do not overreach. Keep proper footing and balance at all times. This enables better control of the machine in unexpected situations.

• Dress properly. Do not wear loose clothing or jewellery. Keep your hair, clothing and gloves away from moving parts. Loose clothes, jewellery or long hair can be caught in moving parts.

• If devices are provided for the connection of dust extraction and collection facilities, ensure these are connected and properly used. Use of these devices can reduce dust related hazards.

4)Machine use and care

• Do not force the machine. Use the correct machine for your application. The correct machine will do the job better and safer at the rate for which it was designed.

•Do not use the machine if the switch does not turn it on and off. Any machine that can not be controlled with the switch is dangerous and must be repaired.

•Disconnect the plug from the power source before making any adjustments, changing accessories, or storing machines. Such preventive safety measures reduce the risk of starting the machine accidentally.

• Store idle machines out of the reach children and do not allow persons unfamiliar with the machine or these instructions to operate the machine. Machines are dangerous in the hands of untrained users.

• Maintain machines. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the machines operation. If damaged, have the machine repaired before use. Many accidents are caused by poorly maintained machines.

•Keep cutting tools sharp and clean. Properly maintained cutting tools with sharp cutting edges are less likely to bind and are easier to control.

• Use the machine, accessories and tool bits etc., in accordance with these instructions and in the manner intended for the particular type of machine, taking into account the working conditions and the work to be performed. Use of the machine for operations different from intended could result in a hazardous situation.

5) Service

• Have your machine serviced by a qualified repair person using only identical replacement parts. This will ensure that the safety of the machine is maintained.

##### SPECIFIC SAFETY RULES FOR palm sander:

If you are working with the sanding machine, always wear ear protector and dust mask.

The sanding machine is not suitable for wet sanding.

Check if the switch is not in position “ON”, before you connect the mains plug to the mains voltage.

Keep the mains leads always away from moving parts of the tools.

Use safety glasses, especially when you sand above your head.

Do not bring pressure to your machine, this will only delay the sanding.

OPERATION INSTRUCTIONS

Choice of the sand-paper

Coarse sand paper removes in general the bigger part of the material and fine sand-paper is used for the finishing. If the surface is not even, then start with rough sand-paper. Further you use middle coarse sand-paper and finally you take fine sand-paper for the finishing.

Installing sand-paper

The sanding machine is provided with a rectangle sand sole. By tightening the sand-paper it does not wear out that fast and so it will wear well longer.

- Pull the mains plug out of the power-point;

- Open the clamp lever, see to the paper being in the middle and look after the perforations in the paper. They have to correspond with those on the sand sole, so that the dust can be exhausted.

- Put the clamping spring back into the original position.

- Repeat the operation to fix the sand-paper on the front side.

- Check all the time if the exhaust gaps are not stopped up when you place the sand-paper, and check if the perforations in the sand-paper correspond with the exhaust gaps.

- Fix the sand-paper as tight as possible

Dust collection feature

This model has self-suck function. It need not connect with vacuum cleaner, you just connect the dust bag with the machine. Check and clean the dust bag timely.

Cleaning

Keep the ventilations slots of the machine clean to prevent overheating of the engine. Regularly clean the machine housing with a soft cloth, preferably after each use. Keep the ventilation slots free from dust and dirt. If the dirt does not come off use a soft cloth moistened with soapy water. Never use solvents such as petrol, alcohol, ammonia water,etc.  These solvents may damaged the plastic parts.

Electrical products should not be disposed of with household waste.

Please recycle where facilities exist. Check with your local authority or retailer for recycling advice.

PS350 exploded view

PS350 part list

| No. | Part list | No. | Part list |
| --- | --- | --- | --- |
| 1 | Bearing 627RS | 20 | Rotor |
| 2 | Switch cover | 21 | Capacitor |
| 3 | Switch | 22 | Fan |
| 4 | Carbon brush holder | 23 | Washer |
| 5 | Copper brush nest | 24 | Balance block |
| 6 | Carbon r brush | 25 | Dust cover |
| 7 | Bearing 607RS | 26 | Screw |
| 8 | Inductance | 27 | Base bracket |
| 9 | Stator | 28 | Bearing 629Z |
| 10 | Right housing | 29 | Clamp steel wire cap |
| 11 | Screw | 30 | Clamp Steel wire |
| 12 | Terminal block | 31 | Aluminum base |
| 13 | Cable clamp | 32 | Sponge base |
| 14 | Screw | 33 | Screw |
| 15 | Cable sleeve | 34 | Washer |
| 16 | Cable & plug | 35 | Spring washer |
| 17 | Dust bag | 36 | screw |
| 18 | Dust bag steel wire | 37 | Wool ring |
| 19 | Dust bag bracket | 38 | Left housing |