Concrete vibrator

CV1000

GB

GB

Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.

TECHNICAL SPECIFICATIONS

| Model | CV1000 |
| --- | --- |
| Rated Voltage | 220-240V ~ 50/60Hz |
| Power | 1000W |
| No load speed | 4000/min |
| Vibrator size | φ35mmX1.5m |
| Net weight | 2.4kg (appliance)+3.5kg(vibrator) |

Over view

1. Locking nut

2. Flexile shaft

3. Shaft joint

4. Switch trigger

5. Switch locking button

General Safety Instructions for Electric Tools

This device is exclusively for private use! It is not fit for commercial use!

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury:

SAVE THESE INSTRUCTIONS

1)Work area

• Keep work area clean and well lit. Cluttered and dark areas invite accidents

• Do not operate machines in explosive atmospheres, such as in the presence of flammable liquids, gases or dust. Machines create sparks which may ignite the dust or fumes.

• Keep children and bystanders away while operating a machine. Distractions can cause you to lose control.

2)Electrical safety

• Machine plugs must match the outlet. Never modify the plug in any way. Do not use any adapter plugs with earthed (grounded) machines. Unmodified plugs and matching outlets will reduce risk of electric shock.

• Avoid body contact with earthed or grounded surfaces such as pipes, radiators, ranges and refrigerators. There is an increased risk of electric shock if your body is earthed or grounded.

• Do not expose machines to rain or wet conditions. Water entering a machine will increase the risk of electric shock.

• Do not abuse the cord. Never use the cord for carrying, pulling or unplugging the machine. Keep cord away from heat, oil, sharp edges or moving parts. Damaged or entangled cords increase the risk of electric shock.

• When operating a machine outdoors, use an extension cord suitable for outdoor use. Use of a cord suitable for outdoor use reduces the risk of electric shock.

3)Personal safety

• Stay alert, Watch what you are doing and use common sense when operating a machine. Do not use a machine while you are tired or under the influence of drugs, alcohol or medication. A moment of inattention while operating machines may result in serious personal injury.

• Use safety equipment. Always wear eye protection. Safety equipment such as dust mark, non-skid  safety shoes, hard hat, or hearing protection used for appropriate conditions will reduce personal injuries.

•Avoid accidental starting. Ensure the switch is in the off position before plugging in. Carrying machines with your finger on the switch or plugging in machines that have the switch on invites accidents.

•Remove any adjusting key or wrench before turning the machine on. A wrench or a key left attached to a rotating part of the machine may result in personal injury.

•Do not overreach. Keep proper footing and balance at all times. This enables better control of the machine in unexpected situations.

• Dress properly. Do not wear loose clothing or jewellery. Keep your hair, clothing and gloves away from moving parts. Loose clothes, jewellery or long hair can be caught in moving parts.

• If devices are provided for the connection of dust extraction and collection facilities, ensure these are connected and properly used. Use of these devices can reduce dust related hazards.

4)Machine use and care

• Do not force the machine. Use the correct machine for your application. The correct machine will do the job better and safer at the rate for which it was designed.

•Do not use the machine if the switch does not turn it on and off. Any machine that can not be controlled with the switch is dangerous and must be repaired.

•Disconnect the plug from the power source before making any adjustments, changing accessories, or storing machines. Such preventive safety measures reduce the risk of starting the machine accidentally.

• Store idle machines out of the reach children and do not allow persons unfamiliar with the machine or these instructions to operate the machine. Machines are dangerous in the hands of untrained users.

• Maintain machines. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the machines operation. If damaged, have the machine repaired before use. Many accidents are caused by poorly maintained machines.

•Keep cutting tools sharp and clean. Properly maintained cutting tools with sharp cutting edges are less likely to bind and are easier to control.

• Use the machine, accessories and tool bits etc., in accordance with these instructions and in the manner intended for the particular type of machine, taking into account the working conditions and the work to be performed. Use of the machine for operations different from intended could result in a hazardous situation.

5) Service

• Have your machine serviced by a qualified repair person using only identical replacement parts. This will ensure that the safety of the machine is maintained.

OPERATION INSTRUCTIONS

1. Insert the shaft joint(3) into the hole of flexile shaft(2), turn the locking nut(1) anticlockwise to fix the vibrator.

2. Press the switch trigger (4) to start the motor. If you would like to use continuous operation, press the locking the switch locking button (5) while the switch trigger (4) is pressed. In order to switch off the motor during continuous operation, press the switch trigger (4) again and then let it go.

3. After the vibrator motor is started, if the vibrator does not start vibration, it is should be knocked until exciting. Then the vibrator can be used.

4. During operation, the motor should be hung on a scaffold or lain flat on hard surface to use. Never let it on the concrete which has not yet set to use. A wood plate which is more than 30cm x30cm can be on the unconsolidated concrete for the need of construction site. Then the motor is put on the wood plate and fixed to use for avoidance that the motor is stuck in the unconsolidated concrete to be damaged.

Attention: the locking nut is hot when running, do not touch.

Maintenance

1. When it is operated, the outside of the motor should be cleaned whenever necessary. Especially the inlet and outlet of the motor must always be kept clear.

2. When it is operated, check if any one of the connecting screw is loose. The place where the vibrator connects with the motor should be cleaned up, and fed some drops of oil, then the connector of the vibrator is covered with a protective sheath.

3. During operation, care should be taken to observe the wear of carbon brush. If should be replaced to keep the motor away from damaging. The carbon brush only be replaced by a technically qualified person.

4. The brushes should be checked periodically and worn-out brushes should be replace in time ,after replacing ,inspect, whether the new brushes can move freely in the brush holder, keep running the motor for 15 minutes to match the contact of brushes and commutator.

5. Keep the ventilation slots clear from dirt, clean off the accumulated dust and oil dirt periodically.

6. During normal operation, if anything happens ,the power supply should be cut off at once and the tool should be checked and repaired

7. Regular inspect all mounting screws and ensure that they are properly tightened, should any of the screws be loosen, Tighten them immediately, failure to do so could result in personal injury.

8. Do not expose the tool to rain , Do not use power tools in damp or wet location, keep work area well lit do not use power tools where is risk to cause fire or explosion.

### Cleaning and Care

• Before cleaning the machine, remove the mains plug from the socket .

• For cleaning purposes, only use a slightly damp but not wet cloth!

• Do not use any abrasive additives (petrol, turpentine etc) as these may damage the plastic housing!

• A mild soap is suitable for cleaning the machine!

• Ensure that no moisture is allowed to get inside the machine!

• After cleaning, wipe the machine with a dry cloth!

Troubleshooting

| Item trouble | Cause | Method |
| --- | --- | --- |
| Motor do not working | 1. the power socked is damaged or loose contacted. 2. the plug is damaged. 3. the cable is worn or broken. 4. the switch is damaged or loose contacted. 5. the carbon brush have been worn. 6. the inner wire has loosed. 7. the stator or rotor is damaged. | 1. repair or replace socked. 2. replace the plug. 3. replace cable. 4. repair or replace the switch. 5. replace the carbon brush. 6. connect the inner wire. 7. replace the stator or rotor |
| Big spark on the carbon brush | 1.the commutator is damaged. 2.the rotor or stator winding is shorted, or rotor winding is opened circuit. 3.carbon brush will be finished | 1.replace the rotor. 2.replace the rotor or stator. 3.replace the new carbon brush |
| The vibrator do not vibrated or vibration is weak | 1.they are dirt ,oil and water in the vibrator head. 2.the o-ring seal has been damaged. | 1.clean and dry, and assemble them again. 2.replace the O-ring seal. |

CV1000 exploded view

.............................................................