Rotary Sander

RS350

GB

Read this operators guide carefully, before using the machine. Ensure that you know how the machine works, and how it should be operated. Maintain the machine in accordance with the instructions, and make certain that the machine functions correctly. Keep this operator’s guide and other enclosed documentation with the machine.

TECHNICAL SPECIFICATIONS

| Model | RS350 |
| --- | --- |
| Rated Voltage | 220-240V ~ 50/60Hz |
| Power | 350W |
| impact frequency | 6000-1300/min |
| Sanding pad size | φ125mm |
| Net weight | 1.42kg |

Over view

1. Sole                                 2. Handle

3. Variable speed switch                   4.Power switch

5. Dust bag connection                    6.Cable sleeve

7. Dust bag

General Safety Instructions for Electric Tools

This device is exclusively for private use! It is not fit for commercial use!

WARNING! Read all instructions. Failure to follow all instructions listed below may result in electric shock, fire and/or serious injury:

SAVE THESE INSTRUCTIONS

1)Work area

• Keep work area clean and well lit. Cluttered and dark areas invite accidents

• Do not operate machines in explosive atmospheres, such as in the presence of flammable liquids, gases or dust. Machines create sparks which may ignite the dust or fumes.

• Keep children and bystanders away while operating a machine. Distractions can cause you to lose control.

2)Electrical safety

• Machine plugs must match the outlet. Never modify the plug in any way. Do not use any adapter plugs with earthed (grounded) machines. Unmodified plugs and matching outlets will reduce risk of electric shock.

• Avoid body contact with earthed or grounded surfaces such as pipes, radiators, ranges and refrigerators. There is an increased risk of electric shock if your body is earthed or grounded.

• Do not expose machines to rain or wet conditions. Water entering a machine will increase the risk of electric shock.

• Do not abuse the cord. Never use the cord for carrying, pulling or unplugging the machine. Keep cord away from heat, oil, sharp edges or moving parts. Damaged or entangled cords increase the risk of electric shock.

• When operating a machine outdoors, use an extension cord suitable for outdoor use. Use of a cord suitable for outdoor use reduces the risk of electric shock.

3)Personal safety

• Stay alert, Watch what you are doing and use common sense when operating a machine. Do not use a machine while you are tired or under the influence of drugs, alcohol or medication. A moment of inattention while operating machines may result in serious personal injury.

• Use safety equipment. Always wear eye protection. Safety equipment such as dust mark, non-skid  safety shoes, hard hat, or hearing protection used for appropriate conditions will reduce personal injuries.

•Avoid accidental starting. Ensure the switch is in the off position before plugging in. Carrying machines with your finger on the switch or plugging in machines that have the switch on invites accidents.

•Remove any adjusting key or wrench before turning the machine on. A wrench or a key left attached to a rotating part of the machine may result in personal injury.

•Do not overreach. Keep proper footing and balance at all times. This enables better control of the machine in unexpected situations.

• Dress properly. Do not wear loose clothing or jewellery. Keep your hair, clothing and gloves away from moving parts. Loose clothes, jewellery or long hair can be caught in moving parts.

• If devices are provided for the connection of dust extraction and collection facilities, ensure these are connected and properly used. Use of these devices can reduce dust related hazards.

4)Machine use and care

• Do not force the machine. Use the correct machine for your application. The correct machine will do the job better and safer at the rate for which it was designed.

•Do not use the machine if the switch does not turn it on and off. Any machine that can not be controlled with the switch is dangerous and must be repaired.

•Disconnect the plug from the power source before making any adjustments, changing accessories, or storing machines. Such preventive safety measures reduce the risk of starting the machine accidentally.

• Store idle machines out of the reach children and do not allow persons unfamiliar with the machine or these instructions to operate the machine. Machines are dangerous in the hands of untrained users.

• Maintain machines. Check for misalignment or binding of moving parts, breakage of parts and any other condition that may affect the machines operation. If damaged, have the machine repaired before use. Many accidents are caused by poorly maintained machines.

•Keep cutting tools sharp and clean. Properly maintained cutting tools with sharp cutting edges are less likely to bind and are easier to control.

• Use the machine, accessories and tool bits etc., in accordance with these instructions and in the manner intended for the particular type of machine, taking into account the working conditions and the work to be performed. Use of the machine for operations different from intended could result in a hazardous situation.

5) Service

• Have your machine serviced by a qualified repair person using only identical replacement parts. This will ensure that the safety of the machine is maintained.

##### SPECIFIC SAFETY RULES FOR ROTARY SANDER:

1. Observe the applicable regulations when sanding painted surfaces.

2. Whenever possible, use a vacuum extractor for dust collection.

3. Remove dust and other waste matter in an environmentally safe way.

4. Take special care when sanding lead based paint.

OPERATION INSTRUCTIONS

Fitting sanding sheets

Make sure that the perforations in the paper correspond with those on the sanding pad. Press the paper firmly on the sanding pad.

Attention: the sanding paper must be fitted firmly on the sanding pad and should not come loose.

Switch on and off

To switch on the machine press the “1” on the switch (A).

To adjust the rotary speed, turn the adjusting wheel (B).

To switch off the machine press the “0” on the switch (A).

Attention: never use the machine for sanding magnesium workpieces.

Dust extraction

The dust extraction outlet together with the dust bag takes care of the dust extraction of the working surface.

Side the dust bag onto the connection . Regularly empty the dust bag to ensure optimum dust extraction

Cleaning and maintenance

The machine does not require any special maintenance.

Regularly clean the ventilation slots.

Electrical products should not be disposed of with household waste.

Please recycle where facilities exist. Check with your local authority or retailer for recycling advice.

\

RS350  exploded view

RS350 part list

| No. | Part list | No. | Part list |
| --- | --- | --- | --- |
| 1 | Screw | 20 | Spring washer |
| 2 | Right housing | 21 | screw |
| 3 | Inductance | 22 | Press board |
| 4 | Stator | 23 | Base |
| 5 | Right wind guide | 24 | Spring washer |
| 6 | Screw | 25 | Screw |
| 7 | Bearing 607 | 26 | Left housing |
| 8 | Carbon brush holder | 27 | Left wind guide |
| 9 | Carbon brush | 28 | screw |
| 10 | Rotor | 29 | Dust outlet |
| 11 | Bearing 609 | 30 | Dust bag steel |
| 12 | Switch | 31 | Steel bag |
| 13 | Switch cover | 32 | Variable speed switch |
| 14 | Fan | 33 | Screw |
| 15 | Washer | 34 | Cable clamp |
| 16 | Balance block | 35 | Cable sleeve |
| 17 | Bearing seat | 36 | Cable & plug |
| 18 | Bearing 6002 |  |  |
| 19 | Washer |  |  |